SECTION 081116 – ALUMINUM DOORS & FRAMES

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes operable aluminum-framed doors for exterior locations.

        - Related Sections include the following:

          - Division 08 Section “Aluminum Windows” for coordination among aluminum fenestration units.

          - Division 08 Section "Aluminum-Framed Entrances and Storefronts" for coordination among aluminum fenestration units.

          - Division 08 Section "Glazing" for additional glazing requirements.

          - Division 08 Section "Glazed Aluminum Curtain Walls" for coordination among aluminum fenestration units.

      - DEFINITIONS

        - Performance class designations according to AAMA/WDMA/CSA 101/I.S.2/A440-17:

          - AW:  Architectural.

        - Performance grade number according to AAMA/WDMA/CSA 101/I.S.2/A440-17:

          - Design pressure number in pounds force per square foot (pascals) used to determine the structural test pressure and water test pressure.

        - Structural Test Pressure:  For uniform load structural test, is equivalent to 150 percent of the design pressure.

        - Minimum Test Size:  Smallest size permitted for performance class (gateway test size) or as specified elsewhere in this section, whichever is more stringent.  Products must be tested at minimum test size or at a size larger than minimum test size to comply with requirements for performance class.  Downsized test reports will not be considered acceptable.

      - PERFORMANCE REQUIREMENTS

        - General:  Provide aluminum doors [and associated fixed window transoms or sidelights] capable of complying with performance requirements indicated, based on testing manufacturer's windows that are representative of those specified, and that are of minimum test size indicated below:

          - Terrace Doors:  48” x 96”.

          - [Fixed Windows:  60” x 99”.]

        - Structural Performance:  Provide aluminum doors [and associated fixed window transoms or sidelights] capable of withstanding the effects of the following loads, based on testing units of the minimum test size specified herein that pass AAMA/WDMA/CSA 101/I.S.2/A440-17, Uniform Load Structural and Uniform Load Deflection Tests:

          - Uniform Load Structural Test:  150 psf Terrace Door [, 225 psf Fixed Window] (positive and negative).

          - Uniform Load Deflection Test:  100 psf Terrace Door [, 150 psf Fixed Window] (positive and negative).

        - [Windborne-Debris Resistance:  Provide glazed doors [and windows] capable of resisting large missile impact from windborne debris, based on the pass/fail criteria as determined from testing glazed doors identical to those specified, according to ASTM E 1886 and testing information in ASTM E 1996 and requirements of authorities having jurisdiction.] <Note to Specifier:  Windborne debris resistance requires the use of Lexan MR10 or Laminated Glass (.090” PVB Interlayer) at the exterior lite.>

      - SUBMITTALS

        - Product Data:  Include construction details, material descriptions, fabrication methods, dimensions of individual components and profiles, hardware, finishes, and operating instructions for each type of aluminum door indicated.

        - Shop Drawings:  Include plans, elevations, sections, details, hardware, attachments to other work, operational clearances, installation details, and the following:

          - Mullion details, including reinforcement and stiffeners.

          - Joinery details.

          - Weather-stripping details.

          - Thermal-break details.

          - Glazing details.

        - Samples for Initial Selection:  For units with factory-applied color finishes.

          - Include similar samples of hardware and accessories involving color selection.

        - Maintenance Data:  For operable door sash, operating hardware and finishes to include in maintenance manuals.

        - Warranty:  Special warranty specified in this Section.

      - QUALITY ASSURANCE

        - Product Qualifications:  In order to confirm that the proposed product(s) conform to the material and performance requirements contained in these specifications, bidders shall include the following with their bid.  Failure to comply with these requirements shall cause the bid to automatically be rejected.

          - Bidder’s Acknowledgement:  Bidders shall include a letter in their bid stating the manufacturer and series (model) number of the product upon which its bid has been based.  Changes in product (manufacturer or series) will not be permitted after the bid.

          - Product Data:  Bidders submitting bids based on products other than the Basis of Design product listed in Paragraph 2.1 must also include the following with their bid:

            - Comprehensive test reports not more than four years old prepared by a qualified testing agency for each product type being used on the project demonstrating compliance with the air, water and structural requirements outlined herein.  Test reports based on the use of downsized test units will not be accepted.

            - Thermal simulations prepared by a qualified independent testing agency for each product type being used on the project demonstrating compliance with the thermal transmittance requirements outlined in Paragraph 2.3.

            - Full size product details showing all frame and sash details, dimensions, thermal break construction, wall thicknesses and joinery.  Details must accurately reflect all glazing and hardware options specified herein.

        - Product Requirements:  For maximum performance, doors for this project must meet both the testing requirements as contained herein and the minimum material requirements specified.  Doors that carry the applicable AAMA rating but do not meet the material thicknesses, depths, etc. shall not be acceptable for use on this project.

        - Installer Qualifications:  An installer acceptable to aluminum door manufacturer for installation of units required for this Project.

        - Source Limitations:  Obtain aluminum terrace doors and windows through one source from a single manufacturer.

        - Product Options:  Drawings indicate size, profiles, and dimensional requirements of aluminum windows and are based on the specific system indicated.  Do not modify size and dimensional requirements.

          - Do not modify intended aesthetic effects, as judged solely by Architect, except with Architect's approval.  If modifications are proposed, submit comprehensive explanatory data to Architect for review.

        - Fenestration Standard:  Comply with AAMA/WDMA/CSA 101/I.S.2/A440-17, "Standard/Specification for Windows, Doors, and Unit Skylights” for definitions and minimum standards of performance, materials, components, accessories, and fabrication.  Comply with more stringent requirements if indicated.

Retain requirements for AAMA or WDMA certification and labeling if required.  Not all manufacturers that test products to demonstrate compliance with AAMA/WDMA testing and performance requirements participate in AAMA's or WDMA's third-party certification program for listing and labeling aluminum windows.

        - Glazing Publications:  Comply with published recommendations of glass manufacturers and with GANA's "Glazing Manual" unless more stringent requirements are indicated.

        - Preinstallation Conference:  If requested, conduct conference at project site to review methods and procedures related to aluminum doors including, but not limited to, the following:

          - Review and finalize construction schedule and verify availability of materials, Installer's personnel, equipment, and facilities needed to make progress and avoid delays.

          - Review, discuss, and coordinate the interrelationship of aluminum doors with other exterior wall components.

          - Review and discuss the sequence of work required to construct a watertight and weathertight exterior building envelope.

          - Inspect and discuss the condition of substrate and other preparatory work performed by other trades.

      - PROJECT CONDITIONS

        - Field Measurements:  For retrofit installations, verify aluminum door openings by field measurements before fabrication and indicate measurements on Shop Drawings.

          - Established Dimensions:  Where field measurements cannot be made without delaying the Work, establish opening dimensions and proceed with fabricating aluminum doors [and fixed windows] without field measurements.  Coordinate wall construction to ensure that actual opening dimensions correspond to established dimensions.

      - WARRANTY

        - Special Warranty:  Manufacturer's standard form in which manufacturer agrees to repair or replace aluminum doors [and fixed windows] that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Failure to meet performance requirements.

            - Structural failures including excessive deflection, water leakage, or air infiltration.

            - Faulty operation of movable sash and hardware.

            - Deterioration of metals or other materials beyond that which is normal.

            - Failure of insulating glass.

          - Warranty Period:

            - Door [and Window]: [Two] [Three] [Five] years from date of Substantial Completion.

            - [Insulated Glazing:  10 years from date of Substantial Completion.]

            - [Painted Metal Finishes:]

              - [Five years from date of Substantial Completion for AAMA 2603 Baked Enamel Finishes.]

              - [Ten years from date of Substantial Completion for AAMA 2604 High Performance Finishes.]

              - [Fifteen years from date of Substantial Completion for AAMA 2605 Superior Performance Finishes.]

- PRODUCTS

      - MANUFACTURERS

        - Basis-of-Design Product:  The basis of design for these specifications is the Series 920i Terrace Door [and Fixed Window] as manufactured by Architectural Window Manufacturing Corporation, Rutherford, New Jersey.

        - Equivalents:  Subject to compliance with all material and performance requirements outlined in these specifications, “or equal” products by other manufacturers will be considered for use subject to review by the Architect.  The Architect’s decision regarding equivalency is final.

      - MATERIALS

        - Aluminum Extrusions:  Alloy and temper recommended by aluminum door manufacturer for strength, corrosion resistance, and application of required finish, but not less than 22,000-psi (150-MPa) ultimate tensile strength, not less than 16,000-psi (110-MPa) minimum yield strength, and not less than 0.125-inch thickness at any location for the main frame and sash.

        - Frame/Sash Depth: [3 ½”] [4 ¼”] [5”] minimum frame depth; 2 ½” minimum sash depth.

        - Fasteners:  Aluminum, nonmagnetic stainless steel, epoxy adhesive, or other materials warranted by manufacturer to be noncorrosive and compatible with aluminum door members, trim, hardware, anchors, and other components.

          - All fasteners must be concealed except where unavoidable for application of hardware.

          - For application of hardware, where required, use non-magnetic stainless steel phillips machine screws.

        - Anchors, Clips, and Accessories:  Aluminum, nonmagnetic stainless steel, or zinc-coated steel or iron complying with ASTM B 633 for SC 3 severe service conditions; provide sufficient strength to withstand design pressure indicated.

        - Compression-Type Weather Stripping:  Provide compressible weather stripping designed for permanently resilient sealing under bumper or wiper action and for complete concealment when aluminum window is closed.

          - Weather-Stripping Material:  Manufacturer's standard system and materials complying with AAMA/WDMA/CSA 101/I.S.2/A440-17.

        - Sliding-Type Weather Stripping:  Provide woven-pile weather stripping of wool, polypropylene, or nylon pile and resin-impregnated backing fabric.  Comply with AAMA 701/702.

          - Weather Seals:  Provide weather stripping with integral barrier fin or fins of semirigid, polypropylene sheet or polypropylene-coated material.  Comply with AAMA 701/702.

        - Replaceable Weather Seals:  Comply with AAMA 701/702.

        - Sealant:  For sealants required within fabricated doors, provide door manufacturer's standard, permanently elastic, nonshrinking, and nonmigrating type recommended by sealant manufacturer for joint size and movement.

      - DOOR

        - Door Type:  Terrace Door (Outswing/Inswing)

        - AAMA/WDMA Performance Requirements:  Provide aluminum terrace doors of performance indicated that comply with AAMA/WDMA/CSA 101/I.S.2/A440-17.

          - Performance Class and Grade:  AW-PG100.

        - Condensation-Resistance Factor (CRF):  Provide aluminum doors tested with insulating glass for thermal performance according to AAMA 1503, showing a minimum CRF of 54.

        - Thermal Transmittance:  Provide aluminum doors with whole-door U-factor and SHGC maximums indicated when simulated in accordance with NFRC 100 and NFRC 200 at the model size of 37 ¾” x 82 ⅜” and glazed with 1” Argon filled sputter coat Low-E (#2) insulated glass using a warm edge spacer.

          - U-Factor: [0.43 3 ½”] [0.45 4 ¼”] [0.46 5”] Terrace Door Btu/sq. ft. x h x deg F or less.

          - SHGC:	0.26 Terrace Door.

        - Air Infiltration:  Maximum rate not more than indicated when tested according to AAMA/WDMA/CSA 101/I.S.2/A440-17, Air Infiltration Test.

          - Maximum Rate: [0.02 outswing] [0.01 inswing] cfm/sq. ft. of area at an inward test pressure of 6.24 lbf/sq. ft.

        - Water Resistance:  No water leakage as defined in AAMA/WDMA referenced test methods at a water test pressure equaling that indicated, when tested according to AAMA/WDMA/CSA 101/I.S.2/A440-17, Water Resistance Test.

          - Test Pressure: 20 percent of positive design pressure, but not more than 12 lbf/sq. ft.   <Note to Specifier:  Twin inswing doors provide reduced water resistance.>

        - Forced-Entry Resistance:  Comply with Performance Grade 10 requirements when tested according to ASTM F 588.

        - Life-Cycle Testing:  Test according to AAMA 910 and comply with AAMA/WDMA/CSA 101/I.S.2/A440-17.

        - Operating Force and Auxiliary (Durability) Tests:  Comply with AAMA/WDMA/CSA 101/I.S.2/A440-17 for operating window types indicated.

      - [INSULATED] <DUAL> GLAZING

        - Construction:  All doors [and fixed windows] shall be factory glazed with [hermetically sealed 1” insulating glass units with a dual seal of polyisobutylene and silicone and a desiccant filled spacer.] <dual glazing>.  [Insulated] <Exterior lite of> glass must be set into a continuous bed of silicone sealant and held in place with removable extruded aluminum snap-in beads.  [Wrap around (marine) glazing which requires the removal and disassembling of the sash for re-glazing will not be acceptable.  Units must be IGCC certified for a CBA rating level.] < Interior glazing shall be wrap around (marine) glazed into a removable access panel.  Access panels shall be hollow extruded sections with minimum wall thickness of 0.062 inches and shall be miter cut and assembled with stainless steel screws for ease of repair.  Tamper resistant security fastenings shall be installed at the bottom of each panel to securely attach panels to sash.  For safety purposes, access panels shall be encased within channels at the top and bottom to prevent the panel from falling out even if the security fastening is removed.>

          - Exterior Glazing:

            - Thickness: [1/8”] [3/16”] [1/4”] [5/16”]

            - Tint: [Clear] [Bronze] [Grey] [Green]

            - Type: [Tempered Glass] [Laminated Glass (.030” PVB Interlayer)] [Laminated Glass (.060” PVB Interlayer)] [Laminated Glass (.090” PVB Interlayer)] <Lexan® MR10 (or equal)>

            - [Coating: [Pyrolitic Low-E (#2 Surface)] [Guardian SuperNeutral 68, Vitro Solarban 60, Viracon VE-2M Low-E (or equal) (#2 Surface)] [Other (Insert requirements or contact AWM for assistance)]]

          - Interior Glazing:

            - Thickness: [1/8”] [3/16”] [1/4”] [5/16”]

            - Tint: [Clear] [Obscure]

            - Type: [Tempered Glass] [Laminated Glass (.030” PVB Interlayer)] [Laminated Glass (.060” PVB Interlayer)] [Laminated Glass (.090” PVB Interlayer)] <Lexan® MR10 (or equal)>

            - [Coating: [Pyrolitic Low-E (#3 Surface)] [Guardian SuperNeutral 68, Vitro Solarban 60, Viracon VE-2M Low-E (or equal) (#3 Surface)] [Other (Insert requirements or contact AWM for assistance)]]

          - [Interspace Content: [Air] [Argon]]

          - [Spacer Type: [Aluminum] [Warm Edge]]

          - <Integral Louver Blinds:  Provide remotely operated horizontal louver blinds in the space between two panes of dual glazing.  Construct blinds of aluminum slats, approximately 5/8-inch wide, with polyester fiber cords, equipped for tilting by standard operating hardware located on inside face of sash.>

        - Opaque Insulated Panels:  Not applicable

      - HARDWARE

        - General:  Provide manufacturer's standard hardware fabricated from aluminum, stainless steel, carbon steel complying with AAMA 907, or other corrosion-resistant material compatible with aluminum; designed to smoothly operate, tightly close, and securely lock aluminum doors and sized to accommodate sash weight and dimensions. Do not use aluminum in frictional contact with other metals.

        - Locks and Latches:  Designed to allow unobstructed movement of the sash across adjacent sash in direction indicated and operated from the inside only.

        - Terrace Doors:  Provide the following operating hardware:

          - Handle:  Interior and exterior [standard color painted] [custom color painted] [plated] forged aluminum handle and escutcheon plates with interior thumb turn [and exterior key lock].

          - Lock:  Mortise lock with deadbolt and latch plus two additional automatically engaging stainless steel locking points at jamb (3 over 103” tall) and top/bottom stainless steel shoot bolts.

          - Stop: [One concealed 90-degree stainless steel door stop.] [One standard color painted surface applied medium duty door closer.]

          - Hinges:  Three adjustable aluminum butt hinges painted to match the door panel and frame.

      - INSECT SCREENS

        - General:  Not applicable.

      - ACCESSORIES

        - Rescue Labels:  Not applicable.

      - FABRICATION

        - Door panel will be fabricated from double tubular sections with mechanically crimped mitered corners and shall be operable in either an outswing or inswing direction.

        - Fabricate aluminum doors [and fixed windows] in sizes indicated.  Include a complete system for assembling components and anchoring doors.

        - Fabricate aluminum doors [and fixed windows] that are reglazable without dismantling sash or ventilator framing.

        - Thermally Improved Construction:  Fabricate aluminum doors [and fixed windows] with an integral, concealed (products with exposed thermal barriers will not be acceptable), low-conductance thermal barrier; located between exterior materials and door members exposed on interior side; in a manner that eliminates direct metal-to-metal contact.

          - All exterior aluminum shall be separated from interior aluminum by a rigid, structural thermal barrier.  For purposes of this specification, a structural thermal barrier is defined as a system that shall transfer shear during bending and, therefore, promote composite action between the exterior and interior extrusions.

          - No thermal short circuits shall occur between the exterior and interior.

          - The thermal barrier shall be INSULBAR® or equal and shall consist of two glass reinforced polyamide nylon 6/6 struts mechanically crimped in raceways extruded in the exterior and interior extrusions.

          - Poured and debridged urethane thermal barriers shall not be permitted.

        - Weather Stripping:  Provide full-perimeter weather stripping for each operable sash and ventilator.

        - Weep Holes:  Provide weep holes and internal passages to conduct infiltrating water to exterior.

        - Mullions:  Provide mullions and cover plates as shown, matching door units, complete with anchors for support to structure and installation of door units.  Allow for erection tolerances and provide for movement of door units due to thermal expansion and building deflections, as indicated.  Provide mullions and cover plates capable of withstanding design loads of door units.

        - Subframes:  Provide subframes with anchors for door units as shown, of profile and dimensions indicated but not less than 0.093-inch- thick extruded aluminum.  Finish to match door units.  Provide subframes capable of withstanding design loads of door units.

        - Factory-Glazed Fabrication:  Glaze aluminum doors [and fixed windows] in the factory where practical and possible for applications indicated.  Comply with requirements in Division 08 Section "Glazing" and with AAMA/WDMA/CSA 101/I.S.2/A440-17.

        - Glazing Stops:  Provide snap-on glazing stops coordinated with Division 08 Section "Glazing" and glazing system indicated.  Provide glazing stops to match sash and ventilator frames.

        - Muntins:  Where shown on drawings, muntins shall be [3/8” deep profiled extruded aluminum applied to the exterior of 1” deep insulating glass.  Roll formed muntins shall not be acceptable.] [3/8” deep profiled extruded aluminum between the lites of dual glazing.  Roll formed muntins shall not be acceptable.] [roll formed aluminum between the lites of 1” deep insulating glass.]

      - FINISHES, GENERAL

        - Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

        - Finish designations prefixed by AA comply with the system established by the Aluminum Association for designating aluminum finishes.

        - Exterior of Door [and Window]:

          - [Class II, Clear Anodic Finish:  AA-M12C22A31 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class II, clear coating 0.010 mm or thicker) complying with AAMA 611.]

          - [Class I, Clear Anodic Finish:  AA-M12C22A41 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class I, clear coating 0.018 mm or thicker) complying with AAMA 611.]

          - [Class I, Color Anodic Finish:  AA-M12C22A42/A44 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class I, integrally colored or electrolytically deposited color coating 0.018 mm or thicker) complying with AAMA 611.

            - Color: [Champagne] [Medium bronze] [Dark bronze] [Black].]

          - [Baked-Enamel Finish:  AA-C12C42R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; Organic Coating:  as specified below).  Apply baked enamel complying with paint manufacturer's written instructions for cleaning, conversion coating, and painting.

            - Organic Coating:  Thermosetting, modified-acrylic enamel primer/topcoat system complying with AAMA 2603 Organic Coating:  Thermosetting, modified-acrylic enamel primer/topcoat system complying with AAMA 2603.

            - Color:  As selected by Architect from [manufacturer’s standard non-mica, non-exotic, non-metallic colors.] [manufacturer’s standard mica/metallic colors.]  (Note:  Exterior color may be different from interior color.)]

          - [High-Performance Organic Finish:  AA-C12C42R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; Organic Coating:  as specified below).  Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturer’s written instructions.

            - Fluoropolymer Two-Coat System:  Manufacturer’s standard two-coat thermocured system consisting of specially formulated inhibitive primer and fluoropolymer color topcoat containing not less than 50% polyvinylfluoride resin by weight; complying with AAMA 2604.

            - Color:  As selected by Architect from [manufacturer’s standard non-mica, non-exotic, non-metallic colors.] [manufacturer’s standard mica/metallic colors.]  (Note:  Exterior color may be different from interior color.)]

          - [Superior-Performance Organic Finish:  AA-C12C40R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; Organic Coating: as specified below).  Prepare, pretreat, and apply coating to exposed metal surfaces to comply with AAMA 2605 and with coating and resin manufacturer’s written instructions.

            - Fluoropolymer [Two-Coat] [Three-Coat] System:  Manufacturer’s standard [two-coat] [three-coat] thermocured system consisting of specially formulated inhibitive primer [and fluoropolymer color topcoat containing not less than 70 percent polyvinylidene fluoride resin by weight; complying with AAMA 2605.] [, fluoropolymer color coat, and clear fluoropolymer top coat, with both color coat and clear topcoat containing not less than 70 percent polyvinylidene fluoride resin by weight; complying with AAMA 2605.]

            - Color:  As selected by Architect from [manufacturer’s standard non-mica, non-exotic, non-metallic colors.] [manufacturer’s standard mica/metallic colors.]  (Note:  Exterior color may be different from interior color.)]

        - Interior of Door [and Window]:

          - [Class II, Clear Anodic Finish:  AA-M12C22A31 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class II, clear coating 0.010 mm or thicker) complying with AAMA 611.]

          - [Class I, Clear Anodic Finish:  AA-M12C22A41 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class I, clear coating 0.018 mm or thicker) complying with AAMA 611.]

          - [Class I, Color Anodic Finish:  AA-M12C22A42/A44 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class I, integrally colored or electrolytically deposited color coating 0.018 mm or thicker) complying with AAMA 611.

            - Color: [Champagne] [Medium bronze] [Dark bronze] [Black].]

          - [Baked-Enamel Finish:  AA-C12C42R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; Organic Coating:  as specified below).  Apply baked enamel complying with paint manufacturer's written instructions for cleaning, conversion coating, and painting.

            - Organic Coating:  Thermosetting, modified-acrylic enamel primer/topcoat system complying with AAMA 2603 Organic Coating:  Thermosetting, modified-acrylic enamel primer/topcoat system complying with AAMA 2603.

            - Color:  As selected by Architect from [manufacturer’s standard non-mica, non-exotic, non-metallic colors.] [manufacturer’s standard mica/metallic colors.]  (Note:  Exterior color may be different from interior color.)]

          - [High-Performance Organic Finish:  AA-C12C42R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; Organic Coating:  as specified below).  Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturer’s written instructions.

            - Fluoropolymer Two-Coat System:  Manufacturer’s standard two-coat thermocured system consisting of specially formulated inhibitive primer and fluoropolymer color topcoat containing not less than 50% polyvinylfluoride resin by weight; complying with AAMA 2604.

            - Color:  As selected by Architect from [manufacturer’s standard non-mica, non-exotic, non-metallic colors.] [manufacturer’s standard mica/metallic colors.]  (Note:  Exterior color may be different from interior color.)]

          - [Superior-Performance Organic Finish:  AA-C12C40R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; Organic Coating: as specified below).  Prepare, pretreat, and apply coating to exposed metal surfaces to comply with AAMA 2605 and with coating and resin manufacturer’s written instructions.

            - Fluoropolymer [Two-Coat] [Three-Coat] System:  Manufacturer’s standard [two-coat] [three-coat] thermocured system consisting of specially formulated inhibitive primer [and fluoropolymer color topcoat containing not less than 70 percent polyvinylidene fluoride resin by weight; complying with AAMA 2605.] [, fluoropolymer color coat, and clear fluoropolymer top coat, with both color coat and clear topcoat containing not less than 70 percent polyvinylidene fluoride resin by weight; complying with AAMA 2605.]

            - Color:  As selected by Architect from [manufacturer’s standard non-mica, non-exotic, non-metallic colors.] [manufacturer’s standard mica/metallic colors.]  (Note:  Exterior color may be different from interior color.)]

- EXECUTION

      - EXAMINATION

        - Examine openings, substrates, structural support, anchorage, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of work.  Verify rough opening dimensions, levelness of sill plate, and operational clearances.  Examine wall flashings, vapor retarders, water and weather barriers, and other built-in components to ensure a coordinated, weathertight door installation.

          - Masonry Surfaces:  Visibly dry and free of excess mortar, sand, and other construction debris.

          - Wood Frame Walls:  Dry, clean, sound, well nailed, free of voids, and without offsets at joints.  Ensure that nail heads are driven flush with surfaces in opening and within 3 inches (76 mm) of opening.

          - Metal Surfaces:  Dry; clean; free of grease, oil, dirt, rust, corrosion, and welding slag; without sharp edges or offsets at joints.

          - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Comply with Drawings, Shop Drawings, and manufacturer's written instructions for installing doors, hardware, accessories, and other components.

        - Install doors [and fixed windows] level, plumb, square, true to line, without distortion or impeding thermal movement, anchored securely in place to structural support.

        - Set sill members in bed of sealant or with gaskets, as indicated, for weathertight construction.

        - Install doors and components to drain condensation, water penetrating joints, and moisture migrating within windows to the exterior.

        - Separate aluminum and other corrodible surfaces from sources of corrosion or electrolytic action at points of contact with other materials.

      - FACTORY TESTING

        - One door [and fixed window] for each seventy-five manufactured shall be randomly selected by the Owner and Architect to be tested at the manufacturer’s facility for air and water infiltration in order to confirm compliance of the project’s windows with the performance requirements contained in these specifications.  Bidders are to include the cost of transportation, food, and lodging for four representatives of the Owner and/or Architect to witness these tests.

      - FIELD QUALITY CONTROL

Field tests for air and water leakage should be specified for significant projects.  Delete this Article if field tests are not required.  If retaining, indicate number of windows to be tested.

Retain first option in paragraph below if Owner engages testing agency.  If authorities having jurisdiction permit Contractor to engage testing agency, retain second option and "Field quality-control test reports" Paragraph in Part 1 "Submittals" Article.

        - Testing Agency: [If desired, Owner will engage] [Engage] a qualified testing agency to perform tests and inspections and prepare test reports.

          - Testing and inspecting agency will interpret tests and state in each report whether tested work complies with or deviates from requirements.

Revise first paragraph below if more stringent field testing is required to suit Project.  According to AAMA 502, Test Method A is the default, unless otherwise indicated.

        - Testing Services:  Testing and inspecting of installed doors [and fixed windows] shall take place as follows:

          - Testing Methodology:  Testing of doors [and fixed windows] for air infiltration and water resistance shall be performed according to AAMA 502, Test Method [A] [B].  Field test pressures and allowable limits shall be as factored by AAMA 502 from those minimums required to determine laboratory compliance with the applicable Performance Class and Grade pursuant to AAMA/WDMA/CSA 101/I.S.2/A440-17.

          - Testing Extent: [One] [Three] <Insert number or description> door [and fixed window] as selected by Architect and a qualified independent testing and inspecting agency.  Doors [and fixed windows] shall be tested immediately after installation.

          - Test Reports:  Shall be prepared according to AAMA 502.

        - Remediate noncomplying doors and retest as specified above.

        - Additional testing and inspecting, at Contractor's expense, will be performed to determine compliance of remediated doors or additional work with specified requirements.

      - ADJUSTING, CLEANING, AND PROTECTION

        - Adjust operating sashes, screens, hardware, and accessories for a tight fit at contact points and weather stripping for smooth operation and weathertight closure.  Lubricate hardware and moving parts.

        - Manufacturer shall clean all glass and aluminum prior to shipment.

        - Protection of newly installed doors [and fixed windows] and/or final cleaning of glass and aluminum to remove any accumulations that may have occurred during the construction period is to be the responsibility of the General Contractor or Owner.

        - Comply with manufacturer’s written recommendations for final cleaning and maintenance.

      - DEMONSTRATION

        - Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain door operating system.

END OF SECTION 081116